| Risk Assessor: | T Downing | Signature: | T Downing | Date: | 19-11-14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: | Reviewed Pat Shammon | Date: | 19.07.2018 |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The 3D printer works by melting plastic and extruding a fine filament which builds in layers to make 3D objects. The heated nozzle moves in 2 horizontal axis while the base plate moves vertically. The machine and it’s moving parts are exposed, there is some risk of entrapment, entanglement or burns if a hand is put inside the machine during use however the stepper motors while being powerful they are small would not cause more harm than a nip, the extruding head heats to approximately 220C and would cause a burn but is significantly less hot than for example a soldering iron. Any process involving plastics above their melting temperature there is some risk of toxic fumes being given off if it is overheated, | PLA MSDS stored here: O:\TT\Health and Safety\Props MSDS |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Entrapment or entanglement of hair or fingers in stepper motors, drive belts, extruder carriage or moving base plate | L1 | Students are informed of risk during training. No loose clothing to be worn, long hair tied back, no jewellery to be worn. Machine only to be used by trained persons. | M2 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Burns from touching extruder head, heated bed, or hot plastic extrusions | L1 | Users instructed during training not to touch extruder head, heated bed, or extruded plastic before it has cooled | M2 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
| . |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Toxic fumes from overheated plastic | L1 | Ensure correct temperature settings are used. | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
| Warning notice advising of temperatures and suitable material types on wall near machine |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Fire if machine left unattended | L1 | Do not leave unattended or printing overnight | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
| Warning notice advising of not leaving on overnight |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Electric Shock | L1 | Ensure leads are in good condition and do not become damaged by being trapped in any mechanism. Annual PAT test required. | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |