| Risk Assessor: | Pat Shammon | Signature: |  | Date: | 17 June 2008 Revised May 2012 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: | Reviewed 7th April 2014 reviewed Jan 2016 reviewed July 2018 |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The Band Saw is used to cut down timber, polystyrene, plastics and thin metals. It cuts straight lines, free hand, and can be adapted to cut angles by adjusting the bed. There are 3 band saws in the department. This risk assessment was written specifically for the large band saw but applies to the two other machines. | GSMD TT Dept Method Statement: Use of Band Saw Jan 2016 |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Pat Shammon |  | March 2017 |  |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Fingers/limbs might be severed or mutilated; hair may be trapped in the machine. | H3 | All 1st year Design Realisation students are trained at the start of the first term. Associated Studies Students are also taught in term 2 and term 3. Only Students signed off as having received Bandsaw Training are allowed to use the machines. The Band saw is in daily use in this department. Most processes take around a minute from start to finish. Intricate filigree work can take up to 15 minutes. Students are shown how to use the machines guards, to limit exposure to the blade. Students are shown the panic button, and the brake. Students are advised to use push sticks to keep their fingers away from the blade and this is also demonstrated. Students are not allowed to use the machine without a tutor being present. Students are taught the correct clothing and safety precautions to wear whilst us ing the machines | L1 | Medium |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| The Bandsawing process creates dust Short term worst case injury Difficulty in Breathing. Long term worst case injury Lung Cancers | H3 | Each Band Saw has its own dedicated dust extract.. Students are instructed to wear Dust Masks and Visors The machine is cleaned down at the end of each 6 week Allocation, and more frequently if necessary | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Prolonged use of the machinery can damage hearing. | H3 | All users are instructed to wear ear defenders. Users have a duty to advise those nearby of the noise hazard so they too can protect themselves. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |