| Risk Assessor: | A Wilson | Signature: |  | Date: | 12.08.2018 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The department has a Techsoft/Exel Routercam 1290 CNC router with a 1.5kW 12,000-14,000 rpm spindle and a vacuum table. The machine is fully enclosed and interlocked: if the enclosure is opened while the spindle is running the whole machine shuts down. | - GSMD Production Arts Method Statement “Using the CNC router” |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Andy Wilson | Scheduled review | 25.11.2019 |  |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Failure of interlocking could lead to contact with the rotating tool | M2 | - Function of interlocks is checked regularly - Interlocks are kept free of dust and debris | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| The machine can produce large quantities of wood dust, which is not all removed by the extractor and requires cleaning up by operator | M2 | - Users are trained to use a vacuum cleaner to remove wood dust before and after removing work from machine - Users are trained to adjust extractor system for maximum efficiency - Extractor is emptied regularly | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |