| Risk Assessor: | Simon Bond | Signature: |  | Date: | 19/07/10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| Loading and unloading of counterweights on the loading and trimming galleries. | GSMD Method Statement |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Simon Bond | Annual Review | 19/07/10 | 1 |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Strain or injury resulting from lifting and placing weights in counterweight cradle. | M2 | Students are not allowed to load or unload weights until they have received training in accordance with GSMD Method Statement, including manual handling. Minimum of two people if number of weights exceeds five, one passing the weights to the other already in position, two hands on weights at all times while moving. Students are rotated frequently to reduce fatigue | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Hazard from falling weights/impact injuries | H3 | Fly floor staff stand clear of bar being loaded. Loaders keep both hands on the weight while moving it. No throwing or ‘bombing’ of weights allowed. Students given time to load with out being pressured. Weights stacked no more than six high on loading gallery. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Trapping fingers between weights | L1 | Weights have special recesses on both sides to prevent finger damage. Students encouraged to wear safety gloves. | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |