| Risk Assessor: | A Wilson | Signature: |  | Date: | 10.08.2018 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The department has a Fein GRIT GXE 3-phase bench mounted deburring machine. A 2.2kW motor drives a 250 x 60mm rotary wire brush at 1500 rpm. The ends of freshly-cut pieces of steel are offered up to the brush to remove sharp burrs caused by cutting. | HSE HSG129 Health and safety in engineering workshops |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Andy Wilson | Scheduled review | 11.2019 |  |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Contact with the rotating brush could cause injury to hands | M2 | - Users are required to wear gloves - Users are taught to use both hands to hold the work, with the hand nearest the machine gripping from underneath - Work less than 200mm in length is not deburred on this machine | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| The machine can eject steel brush strands and burrs which could cause eye or hand injury | M2 | - Users are required to wear gloves and eye protection | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Small work pieces can become caught by the brush and lodged between the brush and the work rest, resulting in hand injury | M2 | - Users are required to wear gloves - Regular checks ensure that the gap between the brush and the work rest is less than 2mm, and that the work rest is firmly attached - Work less than 200mm in length is not deburred on this machine | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |