| Risk Assessor: |  | Signature: | Nick Peel | Date: | July 2015 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| Lifting lanterns (usually to FOH bridges) using gin wheel/ triple purchase hauling line / chain hoist | Loler register |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Steve Huttly | Updating. | Nov 2008 | 1 |
| Steve Huttly | Updating. | Sept 2009 | 2 |
| Steve Huttly | Updating. | Jan 2010 | 3 |
| Steve Huttly | Updating | April 2014 | 4 |
| Nick Peel | Updating | July 2015 | 5 |
| Ben Collins | Review – outstanding updates awaiting approval | 06/02/19 | 5 |
| Ben Collins | Review – outstanding updates awaiting approval | 11/02/20 | 5 |
| Ben Collins | Updating | 30/03/21 | 6 |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Falling objects potential for impact injuries | 3 | Students instructed not to stand directly beneath when raising or lowering lanterns. Students are instructed to wear a hard hat Lanterns are secured using a rope loop underneath the yoke and then over the G clamp. An exclusion zone should be created around the lifting site | 1 | 3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | 3 | 1 | 3 |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Manual Handling potential for back or upper limb strain | 2 | This may relate to carrying lanterns to the auditorium, raising them and hauling them onto the bridge. Students and staff have MH training and lanterns over 15.5 kg are identified with red markers and should be lifted with a triple purchase lifting set or a chain hoist. | 2 | 4 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
| Use trolleys and wheeled flight cases when possible to move heavy lanterns in the auditorium |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Failure of any part of the lifting equipment |  | Use only LOLER rated and tested lifting equipment and inspect it and the lifting point before use |  |  |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |