| Risk Assessor: | A Wilson | Signature: |  | Date: | 22.11.2019 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The department has two pillar drills: a 12-speed 400W Rexon bench-mounted machine with a 16mm chuck in the props workshop, and a 10-speed 560W Fobco 7/8 floor standing machine with a 20mm chuck in the construction workshop. Both machines are subject to a planned maintenance and inspection regime that includes checks before use, weekly logged checks, and annual mechanical and electrical safety inspections. | - HSE HSG129 Health and safety in engineering workshops - BS 4163:2014 Health and safety for design and technology in educational and similar establishments – Code of practice |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Long hair, loose clothing, etc could become entangled in moving parts of the machine | M2 | The method statement requires that: - Long hair and loose clothing be secured so as not to come into contact with moving parts, and that jewellery be removed - No gloves apart from disposable vinyl or latex gloves are worn. Plasters on hands should be covered with disposable gloves | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Chuck keys,broken drills, swarf, work pieces etc., can be violently ejected from the machine | M2 | The method statement requires that: - Chuck keys should be removed immediately after use - The chuck guard is used to contain swarf and possible broken drills - Work pieces are adequately clamped or held in a vice when necessary (see below) - Eye protection and steel toe-capped boots are worn | L1 | M2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Unexpected spinning of hand-held workpieces could cause injuries to hands | M2 | The method statement requires that: - All work should be held in a vice or otherwise secured to the table, with the following exception… - Timber workpieces more than 300mm long being drilled with small-diameter drill bits (i.e. ≤12mm) may be held by hand, or held against a fence behind and to the left of the drill bit | L1 | M2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Sharp edges on drill, work pieces and swarf can cause cuts | M2 | The method statement requires that: - The drill bit is enclosed by a telescopic guard during use - Steel work pieces are deburred immediately after cutting and before any subsequent processing - Swarf is cleared regularly with a brush and disposed of | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| The drill table can slip down, or heavy objects fall from the table | M2 | The method statement requires that: - On the workshop machine, the collar at the base of the elevation rack-and-pinion should be tested weekly to ensure it is tight - The table should be locked off after moving - The table should be kept clear of tools, other work pieces, and other loose items - All users wear steel toe-capped boots | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Contact with cutting fluid, oil, or grease can irritate the skin | L1 | - The method statements requires that: - Disposable vinyl gloves should be worn when using cutting fluid, and hands washed afterwards - When using the construction machine, disposable vinyl gloves should be worn to prevent contact with oil from the gear box, and hands washed afterwards - Steel workpieces should be cleaned of any cutting fluid from the cold saw immediately after cutting and before any subsequent operations | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |