| Risk Assessor: | Pat Shammon | Signature: |  | Date: | 9 September 2008 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: | Reviewed pat Shammon | Date: | April 2019 |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The portable circular bench saw is used to cut timber and plastics. It cuts straight lines and can be adapted to cut angles and compound angles by adjusting the blade and the guide. The risk assessment applies to the saw in the Props Department which is on wheels. |  |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Pat Shammon |  | March 2017 | 1 |
|  |  | April 2019 |  |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Fingers/ limbs might be severed or mutilated, hair may be trapped in the machine | H3 | All 1st year Design realisation students are trained at the start of term. Students are shown how to use the machine guards to limit exposure to the blade. Students are advised to use push sticks to keep their fingers away from the blade. .Students are not allowed to use the machine without a tutor being present until trained. | L1 | Medium |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| The circular sawing process creates dust. Short term worst case injury Difficulty in Breathing Long term worst case injury Lung Cancer | H3 | The Circular Bench Saw is only to be used with a dust extract and with the ventilation gate opened. Students are instructed to wear dust masks and Visors. Machine is cleaned down at the end of each 6 week allocation and more frequently if necessary. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Wood can get jammed in the machine and forcibly ejected back at the operator | H3 | This is more likely if the guards have not been locked off. Setting and locking the guards is part of the pre use check for the machine. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| If the wheels are not locked into place then the saw might move unexpectedly causing an accident | H3 | Checking that the saw bench is firmly fixed down is a pre use check. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |