| Risk Assessor: | Andy Wilson | Signature: |  | Date: | 10.03.2018 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| Two 250mm Dewalt radial arm saws in the construction workshop are used to cut timber and sheet materials to length, and for cutting joints This machine is subjected to fortnightly maintenance checks by staff and an annual compliance inspection by a specialist contractor. | - Safe use of manually operated cross-cut sawing machines (HSE information sheet no 36, 2011) - PUWER - GSMD method statement Radial arm (cross-cut) saws GSMD risk assessment Wood dust |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Andy Wilson | Scheduled review | 15.04.2019 | 1 |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Finger/hand injury from rotating blade | H3 | - Students are taught to keep hands outside a marked danger area - Machines are fitted with blade guards, return springs and DC braking | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Wood dust. Short term worst case injury Difficulty in Breathing. Long term worst case injury Lung Cancers | H3 | - The cross cut saws are connected to the main workshop extractor, and students are taught to use this at all times - Extraction equipment maintained regularly - See GSMD risk assessment Wood dust. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Small pieces of material may be ejected from the machine | H3 | - Eye protection is worn at all times when using these machines - Students are taught to isolate the saws before removing trapped pieces |  |  |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Loud noise for long periods can cause hearing loss | H3 | - Ear defenders are worn when using these machines - Blades are kept sharp to reduce noise | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Work not supported by the fence can be pulled violently by the saw, potentially crushing fingers between work and fence | M2 | Students are taught to place work securely against the fence, or use a jig or fixture, to prevent work from moving | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |