| Risk Assessor: | A Wilson | Signature: |  | Date: | 08.08.2018 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The department has a Baileigh R-M55 single-phase ring roller that is used to roll square mild steel tube up to 50 x 50 x 3mm and round tube up to 48 x 3.2mm. The machine is operated from a remote pedal control with an e-stop. It can be used in a vertical position (work curves upwards in a vertical plane) or horizontal position ( work curves sideways in a horizontal plane). | HSE HSG129 Health and safety in engineering workshops |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Fingers could be trapped and crushed between the rolls and the workpiece | M2 | - Operators are taught to press the e-stop on the pedal control whenever the machine is stopped, and no-one approaches the machine until they have announced they have done so. The operator ensures that everyone is clear before releasing the e-stop to continue - When it is necessary to feed stock into moving rolls, gloves are not worn, and the stock is held at least 1.2m from the end | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| When the machine is tipped from vertical to horizontal or vice versa, it could overbalance, causing crushing injuries | H3 | - Two chain hoists are used to turn over the machine: one on the top lifting eye to take the weight, and one on the bottom rail to change the orientation. This allows the machine to be turned in a controlled manner without anyone near it | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| When the machine is used vertically, long or heavy rolled work could overbalance and fall when being removed from the machine causing impact or crushing injuries | H3 | - When it is necessary to roll long or heavy work vertically in such a way that it is impossible to take its weight above its centre of balance when removing it, a rope is passed through a pulley above the machine and attached to the end of the work before it is rolled out of reach. This allows the weight of the work to be taken on the rope while it is released from the machine - Hard hats are worn whenever using the machine | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Mill scale can flake from the work as it is rolled and cause cuts or abrasions to those handling it | M2 | - Gloves are worn whenever rolled work is handled - Loose flakes are removed from the work by wire brushing after rolling | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Power cable, control cable, and stock and work around the machine could cause trips | M2 | - Operators are taught to route cables carefully and to keep stock and rolled work neatly stacked away from the work area | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |