| Risk Assessor: |  | Signature: | Nick Peel | Date: | April 2017 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| Using a tallescope | Method Statement for use of Tallescopes |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Steve Huttly | Updating | Julyl 2014 | 4 |
| Steve Huttly | Updating | March 2016 | 5 |
| Nick Peel | Updating | April 2017 | 6 |
| Ben Collins | Annual review/ Updated risk ratings | 06/02/19 | 7 |
| Ben Collins | Annual review | 11/02/20 | 7 |
| Ben Collins | Annual Review | 30/03/21 |  |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Mechanical Failure, of access equipment may result in falls | H3 | Tallescopes are inspected and certified annually and visually inspected before use | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Overloading of tallescope | H3 | No excessive weights are carried up the ladders/access equipment. SWL is clearly marked. Students are trained in safe use and encouraged to use a hauling line instead. | L1 | 3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Failure/collapse due to incorrect use or assembly of tallescope | H3 | Students are trained in safe use and assembly and supervised by trained staff. .It is prohibited to move the Tallescope elevated with a person in the cage. | L1 | 3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Overreaching may cause falls | H3 | Students are instructed in safe use of access equipment and always use outriggers on the tallescope. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Falling objects Potential injury to people working below from falling tools etc | H3 | Code of practice: Tools are individually tied to tool belts. Hard-hats are worn on stage if working at height is in progress. Tallescope has a bag to contain loose items. Debris is not allowed to accumulate in the cage of the tallescope | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Unprotected edges and traps and holes in the stage might cause the tallescope to fall over | H3 | Visually insect the area before use and guard edges. Ensure adequate light levels. The tallescope is not moved with a person in the cage. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Operatives could lose their grip climbing up the ladder section and into the cage | H3 | Operatives are trained to maintain 3 points of contact when climbing the ladder. Operatives must alternate climbing up the Tallescope to avoid fatigue | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |