| Risk Assessor: | T Downing | Signature: | T Downing | Date: | 19-11-14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: | Reviewed pat Shammon | Date: | 19.07.2018 |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| The Vacuum Former works by heating a clamped sheet of plastic from above until pliable, then the object to be formed is raised into place using a lever on the side of the machine which seals the vacuum and allows the plastic to be sucked around the object to form the mould. Usually the material used is High Impact Polystyrene, but ABS and acrylic can be used. | HIPS MSDS stored here: O:\TT\Health and Safety\Props MSDS |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Burns from touching heating elements, parts of machine becoming hot during operation or from hot plastic | M2 | Users instructed during training not to touch heating elements and to be aware of hot areas such as hood, element and plastic clamp and be aware of temperature of plastic once it has been heated. Persons informed not to touch melted plastic with fingers and to allow mould to cool once formed. | M2 | M4 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Entrapment or entanglement of hair or fingers in clamps or sliding heater mechanism | L1 | Students informed of risk during training.. Machine only to be used by trained persons. | M2 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Toxic fumes from overheated plastic | L1 | Ensure correct temperature settings are used and machine is not left unattended. | M2 | M2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Fire if machine left unattended | M2 | Do not leave unattended when heating sheets of plastic. Do not cover machine when hot, or during operation. Train users in potential danger of fire. Allow machine to cool for 30mins before placing in storage area or covering with dust cover. | M2 | M4 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Electric Shock | L1 | Ensure leads are in good condition and do not become damaged by being trapped in any mechanism. Annual PAT test required. | L1 | L1 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |