| Risk Assessor: | Andy Wilson | Signature: |  | Date: | 10.03.2018 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Manager: |  | Signature: |  | Date: |  |  |  |  |

| Description of Task: | Associated Documents |
| --- | --- |
| A Harwi “Piranha” 250mm vertical panel saw is used to cut timber-based and other sheet materials. This machine is subjected to fortnightly maintenance checks by staff and an annual compliance inspection by a specialist contractor. | - Horizontal beam and vertical panel saws (HSE information sheet no 03) - PUWER - GSMD method statement Vertical panel saw - GSMD risk assessment Wood dust |

| Risk Assessor | Reason for review | Date | Version |
| --- | --- | --- | --- |
| Andy Wilson | Scheduled review. Added maintenance info to task description. | 15.04.2019 | 1 |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Finger/hand injury from rotating blade | H3 | - Students are taught to use both hands on the machine head when cutting - Machine blade is fully enclosed, and the motor is mechanically braked | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Wood dust. Short term worst case injury Difficulty in Breathing. Long term worst case injury Lung Cancers | H3 | - This machine is connected to its own dust extractor, and students are taught to use this at all times - See GSMD risk assessment Wood dust. | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Small pieces of material may be ejected from the machine | H3 | - Eye protection is worn at all times when using this machine - The riving knife is set up in compliance with HSE guidelines | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Damage to hearing from noisy processes | H3 | - Ear defenders are worn when using this machine - Blades are kept sharp to reduce noise | L1 | M3 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |

| Consequence |  | Likelihood |  | Risk |  |
| --- | --- | --- | --- | --- | --- |
| Describe the Hazard and how it might cause harm | Rating L1,M2,H3 | Existing Control Measures and Justification | Rating L1,M2,H3 | Consequence X Likelihood L1,L2,M3,M4,H6,H9 |  |
| Muscular/skeletal injury from mishandling of large and/or heavy materials | M2 | Students are taught never to lift full sheets on their own, but to get help in taking them from storage, placing and removing them | L1 | L2 |  |
| Additional Control Measures - Suggestions to be written in the box below and submitted for review |  |  | Revised risk rating (residual risk) |  |  |
|  |  |  | Consequence | Likelihood | Risk |
|  |  |  | Action by |  | Date |