Method Statement

Activity	Use of the Belt Sander/Linnisher

Location	Props Workshop

Staff allowed	Members of the following departments once trained:

Props,  Props Associated Studies, Stage managers

Students allowed	Year Ones 	As part of lessons only

Year Two 	Allowed under supervision

Year Three	Allowed under supervision

# Risk Rating	B (second person in the area member of staff nominally supervising)

Date of creation/revision of COP	May 09

Reviewed:	March 28 2014 (Pat Shammon) Sept 15 Jan 16 March 17 July 2018  Nov 2019( Pat Shammon)

Tasks, Associated Hazards, Action to be taken to avoid hazards

1.	Approved Tasks	Sanding timber.

- Before Use.

- Do not use the machine if any checks are unsatisfactory

- Disconnect machine from mains before adjusting or dismantling

- Make sure the belt is not torn or loose if it is ask lecturer or technician to change the belt.

- check guards and lower them so that only the required amount of belt is 	exposed

- check  the area is clear of hazards or obstructions

- ensure any controls are accessible

- This machine is for wood only Metal sparks could ignite in the dust bag.

3.	During use

- concentrate on the task

- Hold the work firmly on the table

- Never wear gloves

- Do not sand pieces that are too small to be safely supported

- Avoid awkward hand positions where a sudden  slip could cause a hand to 	move into the sanding belt

4.	After use

- Switch off the sander

- Clear up any scrap or excess sawdust

6.	Noise

- This machine is not normally noisy when being used. (102.1 dB(A)).

7.	P.P.E. and clothing

- Clothing should not be loose or likely to be caught in the machine

- Long hair should be tied back or in a hat

- Remove bracelets bangles scarves necklaces and rings

- You must wear eye protection

- Dust Mask must be worn

.