Method Statement

Activity	Using Chop Saw

Location	Props Workshop

Staff allowed	Any staff when trained

Users must be over 18 years of age

Students allowed	Year Ones 	During class teaching only (supervised)

Year Two 	Allowed under supervision

Year Three	Allowed under supervision

# Risk Rating	A	Staff member actively supervising work

Rating can be reduced to B (Second person in the area, member of staff nominally supervising  if student signed off as “able to take responsibility” on Skills Record Sheet

Special notes	Users must be over 18 years of age

Date of revision:	May 2009

Reviewed:	March 28 2014 Jan 2016 March 2017 (Pat Shammon)

1.	Approved Tasks	sawing lengths of timber

### Cutting narrow pieces of board

Use as a mitre saw

2.	Before Use

Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

- ensure dust extraction running

- check guard is secure and undamaged

- visual check on electric cable

- visual check on fence for damage and security

- check any tools for maintenance or adjustment have been removed from the machine

- check floor around the machine is clear of trip hazards

- check bench is clear of obstructions

- do not use if a get in or out  is taking place

- let the blade stop spinning before lifting it from the cut

- allow the machine to speed up before cutting the first piece

3.	During use

- concentrate on the task

- hold work piece firmly against the fence

- keep hands away from the blade

- do not pull the saw too quickly, let the machine do the work

- make sure you are in control of the movement of the machine

- if timber gets behind the machine, leave in position, turn machine off immediately and wait for the blade to stop before removing timber

4.	After use

- ensure machine is switched off

- sweep up any mess produced

- put offcuts in the bin or rack

5.	Noise

- This machine is noisy when being used.

- You must wear ear protection.

- Other staff and students in the area may choose to wear ear protection, warn them if you have a lot of cutting to do.

6.	P.P.E. and clothing

- clothing should not be loose or likely to be caught in the machine

- long hair should be tied back or in a hat

- remove bracelets, bangles scarves necklaces and rings

- you must wear eye protection

- you must wear ear protection