Activity	Using the cold saw

Location	Construction Workshop

Staff allowed	Any staff after induction by construction staff

Users must be over 18 years of age

Students allowed	All students, during 1st year lessons and when signed off in Skills Record

# Risk Rating

Special notes	Use of the get in hoist isolates this machine

Date of creation/revision of COP 	Created 19.02.2017 (AW)

Reviewed 11.19 (AW)

1.   Approved Tasks

Cutting steel and aluminium to length

Cutting other metals as approved by staff

2.	Before Use 	Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

    - check blade guard is secure and undamaged

    - visual check on electric cable

    - check vice for damage and security

    - check that there is sufficient coolant/lubricant in the reservoir and that the pump is turned on

    - check that the lubricant return path isn’t blocked by swarf

    - check any tools for maintenance or adjustment have been removed from the machine

    - check floor around the machine is clear of trip hazards

    - if cutting mitres, check that the blade will clear all parts of the vice, and adjust if necessary

    - ensure the stock is adequately supported on both sides of the machine

3.	During use

    - concentrate on the task

    - ensure stock is firmly clamped into the vice

    - wait for lubricant to flow before starting to cut

    - pull the saw down steadily though the material

    - do not pull the saw too quickly - let the machine do the work

    - clear the lubricant return path of swarf when necessary

    - if material gets behind the machine, leave in position, turn machine off immediately and wait for the blade to stop before removing material

4.	After use

    - deburr work promptly

    - clear up spillages of lubricant

    - put offcuts in the waste bin or relevant recycling bin

    - reset the machine to 90 degrees if finished with mitre

5.	Noise     Cutting some materials can be noisy – you and others nearby may need to wear ear protection.

6.	P.P.E. and clothing

    - clothing should not be loose or likely to be caught in the machine

    - long hair should be tied back or in a hat

    - some sections (e.g. flat steel held vertically) can cause splashing of lubricant – if necessary wear a visor or mask

    - you must wear eye protection

    - you may need to wear ear protection