Method Statement

Activity	Use of Get In Hoist

Location	Scenic Workshop

Staff allowed	Members of the following departments once trained:

Scenic Workshop, Props, Production Management, Contractors

Students allowed	Year Ones 	Under supervision

Year Two 	Under supervision

Year Three	Under supervision

# Risk Rating     A     Staff Member actively supervising

Special notes	The weight limit of the hoist is 500kg

The weight limit of the gallery is 3 tons

A minimum of two people are required for this task, one at floor level, one at street level.

The gallery should not be used as a storage area

Date of creation:		4th September 2006

Reviewed:		1st April 2014

Tasks, Associated Hazards, Action to be taken to avoid hazards

1.	Approved Tasks	Raising or Lowering Materials and Waste

2.	Before Use. 	Do not use the hoist if any checks are unsatisfactory

Report any faults or damage to the Production Manager on Maintenance Request Sheet

Do not attempt any maintenance yourself

### Floor level person

ensure that the area below the hoist is clear of obstructions

ensure that no one else is working in the area

ensure that the area is not too noisy for communication to the street level person

ensure that the street level person is at the window

###### Street level person

ensure that opening the window will not cause a hazard to the public

shout to the floor level person to open the window (or use walkie talkies)

#### 3	Preparation

##### Floor level person

lower street level window using control box marked ‘window’

press GREEN isolator button on box marked ‘immobilisor’ to immobilise all workshop power except that to the hoist

take pendant controller to position where you can see but will not be beneath the load

###### Street Level Person

set out street barrier/bollards to protect members of the public

put on safety harness after checking that it is undamaged and clip to safety running line

having checked below, open the safety gates. Ensure the gates are secure and will not swing closed whilst you are working.

select the best lifting accessory for the task

check that the condition of lifting accessories is satisfactory

4.	Items in from Street level

The floor level person should drive the hoist along the beam until it is over the gallery and lower the hook under instruction from the street level person

the street level person should attach the item to be moved onto the hoist and instruct it to be lifted slightly to check carefully that it is in balance

the floor level person should then raise the load to clear the gallery floor, drive the hoist back along the beam and lower the load

5.	Items out from Workshop level

the floor level person should position the hoist over the load and lower the hook

they should attach the item to be moved onto the hoist and lift it slightly using the hoist to check carefully that it is in balance

they should then raise the load to clear the gallery floor, drive the hoist back along the beam and lower the load under instruction from the street level person

6. 	During the task

concentrate on the task

be very conscious of the public and their safety

do not lean out over the platform edge

never leave the platform unattended with the safety gate open

do not overload the hoist

use the hoist to do the work. This includes running it out into the street when this will help

7.	After use

Street Level Person

Close and bolt safety gate

store barriers from the street on the gallery

remove and hang up safety harness

from outside, instruct the floor level person to raise the window and wait to see it close safely

Floor level person

run the hoist right back to the end of the beam (the window will not raise if this is not done)

switch power back to scenic workshop by pressing the RED button on the box marked ‘immobilisor’

once instructed by the street level person,  raise the window

restore power to the workshop by turning the power key (in the corner by the fuse boxes) to the right. Check that it has activated by running a machine

hang up lifting accessories

store hoist controller out of the way

remove lowered item to the area it is required in

8.	Noise

No protection required

9.	P.P.E. and clothing

safety harness to be worn by street level person and clipped to the running line

you must wear steel capped boots

hard hats must be worn by anyone in the scenic workshop while the hoist is being used

you may wish to wear gloves