METHOD STATEMENT

Activity	Using the Portable Circular Bench Saw

Location	Props Workshop

Staff allowed	Members of the following departments once trained:

Workshop, Props, Production Management

Students allowed	Year Ones 	During class teaching only (supervised)

Year Two 	Allowed under supervision

Year Three	Allowed under supervision

# Risk Rating		B (Second Person in the area, member of staff nominally supervising)

Special notes	Users must be over 18 years of age

Date of creation/revision	4th October 1999. April 2014 Dec 15 April 2019 Pat Shammon

Tasks, Associated Hazards, Action to be taken to avoid hazards

- Approved Tasks

    - Ripping lengths of timber

### Cutting sheet timber

    - Using as a mitre saw

### Cutting sheets of plastic or plastic coated material

    - Cutting of other sheet material as approved by Staff

- Before Use.

- Do not use the machine if any checks are unsatisfactory

- Isolate machine from mains before adjusting or dismantling

- ensure dust extract running

- check guard is secure and undamaged

- check that the floor around the bench saw is clear, particularly the area where you will walk

- ensure that the take-off table is clear

- check that a push stick is available (min length 300mm)

- visual check of the riving knife for damage and proximity to blade (4-5 mm clearance)

- check that the crown guard is securely fitted

- using the blade height adjuster, position guard to approx. 12mm above the material

- check any tools for maintenance or adjustment have 	been removed

- ensure that the ON/OFF buttons are accessible

- if using the cross cut fence ensure that the groove is clear

- if using old timber ensure that screws, nails etc. have been removed

- Check that the guard adjusting levers are in the down position

- The bench saw is capable of cutting 8x4 sheets if it is positioned on its marks. However if you have several sheets to cut ask if the wall saw in construction is available.

- Make sure the wheels on the saw are locked in place  before you offer up any timber.

3.	During use

- concentrate on the task

- do not feed material through too quickly, allow the machine to do the work

- do not leave the machine running unattended

- get assistance if cutting large or heavy sheets of material two people for an 8x4 sheet

- use push sticks when working within 300mm in front of blade or 150mm either side

##### turn machine off and wait for blade to stop before removing trapped pieces

4.	After use

- ensure that the machine is switched off

- switch extractor off if no further use intended

- sweep up any mess produced

- put off cuts in the bin or rack

- clear out saw dust from under machine after major amounts of cutting

- empty dust extractor after major amounts of cutting 	(seek advice from staff)

5.	Noise

- This machine is noisy when being used.

- You must wear ear protection

- If you are about to do a lot of cutting then warn others so they may choose to wear ear protection.

6.	P.P.E. and clothing

- Clothing should not be loose or likely to be caught in the machine

- Long hair should be tied back or in a hat

- Remove bracelets, bangles, scarves, necklaces and rings

- You must wear eye protection

- Mask must be worn