Activity	Radial Arm Saw / Cross Cut Saws

Location	Construction Workshop

Staff allowed	Any staff after induction by construction staff

Users must be over 18 years of age

Students allowed	All students, during 1st year lessons and when signed off in Skills Record

# Risk Rating

Special notes	Use of the get in hoist isolates this machine

Most recent revision: 	April 2019 (AW – minor edits)

1.   Approved Tasks

`	Cross cut sawing lengths of timber and board

Other machining as approved by staff

Cutting other materials as approved by staff

2.	Weekly checks – to be entered in the log 	Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

    - visual check on electric cable

    - check function of on-off switches and DC injection braking

    - check function of work light

    - check that the nose guard: approx. 25mm above bench

    - check function of inner and outer scissor guards

    - check function of return spring

    - check blade height – teeth just below table level

    - check blade for missing or damaged teeth and replace blade as necessary

    - dust extractor ports are unobstructed and clear of debris

    - check that the area around and behind the saw is clear of sawdust and offcuts

3.	Checks before use – do not use until all checks are good

- check function of on-off switches and DC injection braking

- check that the nose guard will be less than 10mm above the top of the work

- check function of inner and outer scissor guards

- check function of return spring

- check blade height – teeth just below table level unless required to be higher

- check that the floor around the machine is clear

- check that the dust extractor ports are clear and unobstructed

4.	During use

    - turn on the dust extractor

    - concentrate on the task

    - hold work piece firmly against the fence

    - keep hands outside the area marked in red

    - push the saw right back and hold it there before starting the motor

    - allow the machine to reach full speed before cutting

    - do not pull the saw too quickly - let the machine do the work

    - make sure you are in control of the movement of the machine

    - do not leave the machine running unattended

    - if material gets behind the machine, leave in position, turn machine off immediately and wait for the blade to stop before removing material

5.	After use

    - ensure machine is switched off

    - sweep up any mess produced

    - put offcuts in the bin or rack

    - reset the machine if height or angle altered

    - close dust extractor ports and switch dust extractor off if no further use intended

6.	P.P.E. and clothing

    - YOU MUST WEAR EYE PROTECTION

    - YOU MUST WEAR EAR PROTECTION

    - CLOTHING SHOULD NOT BE LOOSE OR LIKELY TO BE CAUGHT IN THE MACHINE

    - LONG HAIR SHOULD BE TIED BACK OR IN A HAT