| Method Statement |  |
| --- | --- |
| Activity | Using the Skyjack SJ 111 |
| Location | Performance venues, rehearsal rooms and workshops. Indoor use only |
| Staff allowed | Members of any department once IPAF Mobile Vertical 3A trained. |
| Students allowed | None |
| # Risk Rating | # A Staff Use only (qualified to IPAF Mobile Vertical 3A trained.) |
| Special notes | The SJ 111 can only be used on a flat floor or a rake up to the limits of its adjustment (about 3%) |
|  | ### No lone working. There must be two people present when the Skyjack SJ 111 is being used. Both must be familiar with the operational controls & Method Statement. |
|  | ### The SKYJACK SJ 111 is not to be used by students. It may be moved whilst elevated by staff qualified to IPAF Mobile Vertical 3A level) but a banksman should be present to ensure the safety of anyone in the movement zone. |
| Date of creation: | 06/07/2012 by Steve Huttly |
| Reviewed: | Feb 2020 – Charles Haines |

| Tasks, Associated Hazards, Action to be taken to avoid hazards |  |
| --- | --- |
| 1 | Approved Tasks Indoor work at high level for one person |
|  | #### The Skyjack SJ 111 MUST NOT be used by students |
| 2 | Before Use. |
|  | Carry out pre use inspection as specified on the machine and do not use machine if any faults are found. Record the inspection by signing the check sheet. |
|  | Report any faults, damage or maintenance carried out to Performance Venues staff. |
|  | Ensure the Skyjack SJ 111 has a valid inspection certificate covering the period of proposed use. |
|  | Visually assess the work area for potential hazards e.g. unguarded edges, holes, traps, doors and surface irregularities. |
|  | Check that the work area is capable of supporting the load. E.g. the floor in the props area of Silk St theatre is not. |
|  | The Skyjack weighs 1170kg |
|  | ##### Disconnect the Skyjack SJ 111 from mains before use. |
| 3 | Driving The Machine. |
|  | Before moving the Skyjack SJ 111: |
|  | Check there are no obstructions or people in the way. |
|  | Drive the machine carefully and in ‘Crawl’ setting only. Keep monitoring all perimeters of working area and listen for instructions from Banksman. |
| 4 | Elevating the Skyjack SJ 111 |
|  | Position the Skyjack SJ 111 where you want to work. |
|  | SJ 111 is clear of hazards or obstructions |
|  | Restrict access to the work zone to authorised staff only. |
|  | Clearly announce that you are about to use the Skyjack SJ 111 and ensure that anyone working in the venue is wearing a Hard Hat. |
| 5 | During use; |
|  | The person in the cage should wear a hard hat with a chin strap and should look upwards while elevating. |
|  | The person in the cage should shout ‘hoist moving’ before each movement. |
|  | All tools should be secured to the operator with lanyards & other loose items required for the work in hand should be stored in the tool tray. Do not allow debris to accumulate. |
|  | Check what is above and below you and that it is safe to do so before moving. |
|  | The Skyjack SJ 111 must not be used as a crane to lift large materials or structures, but can be used to lift an operator, tools and small materials for the immediate task provided the safe working load of 227kg [or two people] is not exceeded and everything fits safely inside the cage. |
|  | The Skyjack SJ 111 must not be used by students. |
|  | In case of an emergency, the platform can be lowered using the emergency button on the control panel or in the event of total power failure using the pressure release valve.) |
| 6 | After use: |
|  | Ensure that the charging cable is plugged in and that the hydrogen gas given off during charging can safely dissipate to guard against the risk of explosion. No smoking or naked flames allowed on stage if Skyjack SJ 111 is charging. |
|  | Ensure that the platform and tool tray are left empty and clean. |
|  | The platform should always be left in the fully down position with the key removed. |
| 7 | Periodic checks to be done before each usage period; |
|  | In addition to the pre use inspection specified on the machine. |
|  | Disconnect machine from mains before adjusting or dismantling. |
|  | Check for pump noise, a noisy pump indicates a clogged oil inlet screen filter |
|  | Perform a visual check on the electric cable and plug |
|  | Check visible parts for correct operation and function |
|  | Check that moving parts operate freely and there are no breakages. |
|  | To avoid damage; - care should be taken to avoid impact and corrosive substances. |
|  | ###### To avoid risk of electric shock do not allow fluids to come into contact with electrical components |
| 8 | Noise |
|  | Whilst the Skyjack SJ 111 presents no noise hazard, surrounding sources of noise should be controlled during its use to enable clear communication. |
| 9 | Light |
|  | Ensure there is a sufficient level of light to allow safe working. When focussing ensure the stage doesn’t go to blackout when moving the Skyjack SJ 111 in any dimension. |
|  | . |
| 10 | P.P.E. and clothing |
|  | Anyone working in the performance venue with the Skyjack SJ 111 must wear a hard hat and safety footwear. |
|  | The operator must ensure that all tools are secured on a lanyard and that only loose items for the work in hand are carried in the cage or tool tray. |
|  | The person on the platform must wear a hard hat with a chin strap. |
|  | Wear eye protection, gloves and full body covering when checking acid levels (to be done by staff and service personnel only) |