Method Statement

Activity	Use of two part foam and other mould making  and casting chemicals  including contact glue and epoxy.

Location	Props Department

Staff allowed	Members of the following departments once trained: Props and Construction

Students allowed	Year Ones 	Only under supervision

Year Two 	With permission

Year Three	With permission

# Risk Rating

# B	Second person in the area, member of staff nominally supervising

Date of creation/revision:		May 09

Reviewed: March 28 2014	 (Pat Shammon) Jan 2016  March  April 2019 (Pat Shammon)

Tasks, Associated Hazards, Action to be taken to avoid hazards

1.	Approved Tasks	A broad range of prop making activities. Discuss your plans with the Prop Making Lecturer before you start; there may be a solution which avoids using 	chemicals.

.Before Use.

- Read the COSHH  data sheets before use.

- Download and read COSHH sheets for any new chemicals. Store them in the COSHH folder.

- Do not use the chemicals if any checks are unsatisfactory ie missing lids or liquids thicker than expected.

- Work should  be done in the Vapour Extract Machine. Seek advice if it won’t fit in.

- Put fresh paper down to contain any drips

- Make sure cups, sticks, scales, gloves, timer and all chemicals are to hand 	before you start.  Make sure the lids to the chemicals haved not gummed up 	solid.

- ensure safety measures in place e.g. vapour extract running

- visual check for damage

- check clear of hazards or obstructions

- During use

- concentrate on the task

- use the chemicals as instructed

- Ensure  the correct  fire extinguisher – one for containing a chemical fire,  is to hand.

- What NOT to do

  - do not adjust the proportions of the chemicals

- After use

●		Make sure lids are fitted securely, clean off excess chemicals

- cleari up

- Report any empty containers to the Prop Making Lecturer or technician. dispose of contaminated brushes etc as directed.

5.	P.P.E. and clothing	Vapour Mask

Visor/Goggles

Gloves

Disposable aprons/suit