Activity	Using pillar drills

Location	Construction workshop & props workshop

Staff allowed	Any staff after induction by construction or props staff

Students allowed	-  after induction by relevant staff: induction is specific to each machine

# Risk Rating	 B	Second person in the area, member of staff aware of activity

Special notes	Users must be over 18 years of age

Date of creation/revision of COP 	Created 09.12.2019 (AW)

Revised 26.03.2021 (AW)

1.     Approved tasks

- Drilling and reaming holes in timber, metals, and other materials as agreed with relevant staff

2.	Weekly checks 	Checks to be completed when machine is brought into use and weekly whilst in use. Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

- visual check on cables including plugs

- check function of on-off switch and emergency stops

- check function of rotation direction switch where fitted

- check rotation switch is in default clockwise position

- check gear lever is set to default “slow” position where fitted

- check function of chuck guard

- check condition of belts

- check that belt covers are securely fastened

- check table and head locks are secure

- check oil level where relevant and top up if necessary

- check drill table and area around drill are clear of swarf, oil, tools, and any other debris

3.	Before use

- repeat weekly checks

    - check that power cable is safely routed

    - if the construction machine has been moved, ensure brakes are on

    - ensure metal work pieces have been deburred and cleaned of cutting fluid

    - push E-stop on front of machine in while changing drill bits or adjusting table height

    - select a suitable drill bit

    - for holes larger than 11mm in steel, start with a 6mm pilot and work up in 6mm max increments

    - ensure drill bit is securely mounted in chuck and chuck key has been removed

    - ensure drill bit is sharp and undamaged

    - select suitable speed using belts and/or gear lever

    - if belts are changed, ensure belts are tensioned and covers are securely closed

    - hold work securely in vice, or otherwise fix to table

    - timber work pieces ≥ 300mm long may be hand held when using a drill bit ≤ 12mm diameter

    - support long work with a roller stand or equivalent

    - see section 7 – PPE and clothing

4.	During use

- load the work into the vice before starting the machine

    - start the machine before lowering the drill bit to contact the work

    - feed the drill bit steadily and without forcing it

    - use a pecking technique / interrupted feed to break up and clear swarf from deep holes

    - turn the machine off before changing work pieces, applying cutting fluid, or clearing swarf or chippings

    - push in the E-stop before changing drill bits

    - deburr newly drilled holes promptly

    - push in E-stop or isolate machine from power supply when leaving unattended

5.	At the end of the day:

    - isolate machine from power supply and push in E-stop

    - clean machine and surrounding area of swarf, chippings, and cutting fluid

    - ensure tools and cables are tidied and stored

    - park machine in storage position if it has been moved

6.	Noise  is not normally a problem with this machine, but some jobs may require use of ear protection

7.	P.P.E. and clothing

    - clothing should not be loose or likely to interfere with the work

    - long hair should be tied back or in a hat

    - jewellery on hands or wrists should be removed

    - you must wear eye protection when the machine is running

    - you must wear steel toe-capped boots

    - you must not wear gloves other than disposable vinyl or latex gloves when using this machine