Activity	Using the deburring machine

Location	Construction workshop

Staff allowed	Any staff after induction by construction staff

Students allowed	-  after induction by construction staff

# Risk Rating	 B	Second person in the area, member of staff aware of activity

Special notes	Users must be over 18 years of age

Date of creation/revision of COP 	Created 11.08.2018 (AW)

1.     Approved tasks

Deburring mild steel after cutting

2.	Weekly checks 	Checks to be completed when machine is brought into use and weekly whilst in use. Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

- visual check on accessible cables including C-form plug

- check function of on-off switch

- check machine is firmly bolted or clamped to bench

- check gap between brush and work rest is less than 2mm

- check condition of brush

- check condition of polycarbonate top guard

- check condition of work rest

- check work rest bolts are tight

3.	Before use

- repeat weekly checks

    - ensure gloves, eye, and ear protection are available

    - ensure the area around the machine is clear

4.	During use

    - gloves and eye and ear protection must be worn when using the machine

    - work less than 200mm in length should not be deburred on this machine  unless approved by staff

    - aluminium should not be deburred on this machine

    - if it is necessary to relocate the machine (eg for very long work pieces), it must be securely clamped to a suitable bench

    - isolate machine from power supply when leaving unattended

5.	At the end of the day:

    - isolate machine from power supply

6.	Noise

- others around you may wish to wear ear protection

6.	P.P.E. and clothing

    - clothing should not be loose or likely to interfere with the work

    - long hair should be tied back or in a hat

    - you must wear gloves

    - you must wear eye protection

    - you must wear ear protection