Activity	Using the ring roller (section roller)

Location	Construction workshop, paintshop, Silk Street stage

Staff allowed	Any staff after induction by construction staff

Students allowed	-  after induction by construction staff

# Risk Rating	 B	Second person in the area, member of staff aware of activity

Special notes	Users must be over 18 years of age

Date of creation/revision of COP 	Created 08.08.2018 (AW)

1.     Approved tasks

Rolling curves in mild steel sections up to and including 50 x 50 x 3mm RHS and 48.3 x 3.2mm CHS

2.	Weekly checks 	Checks to be completed when machine is brought into use and weekly whilst in use. Do not use the machine if any checks are unsatisfactory. Isolate machine from mains before adjusting or dismantling

- visual check on accessible cables including C-form plug

- check function of on-off switch and speed selection switch

- check condition of pedal control cable and connector

- check function of pedal switches

- check function of emergency stop switch on pedal control

3.	Before use

- repeat weekly checks

    - check that power and control cables are safely routed

    - prepare pulley and rope if needed

    - ensure gloves and hard hats are available

    - if the machine must be moved from vertical to horizontal use or vice versa, two chain hoists should be used – one to take weight and one to change orientation

4.	During use

- the E-stop should be applied whenever the machine is not being operated

    - the machine operator should announce when the E-stop is being applied or released

    - others should wait until the E-stop is applied before approaching the machine

    - hard hats must be worn when using the machine

    - gloves should be worn when handling rolled work

    - gloves shoud not be worn when feeding work into moving machine

    - when feeding into moving machine, work must be held at least 1.2m from end

    - ensure stock steel and rolled work are neatly stacked away from the work area

    - isolate machine from power supply when leaving unattended

5.	At the end of the day:

    - isolate machine from power supply

    - tidy stock and rolled work to minimise trip hazards

    - do not leave work in the machine

6.	Noise  is not a problem with this process

6.	P.P.E. and clothing

    - clothing should not be loose or likely to interfere with the work

    - long hair should be tied back or in a hat

    - you must wear a hard hat

    - you must wear gloves when handling rolled work