MATERIAL SAFETY DATA SHEET

_________________________________________________________________________________________________________

1.	IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY

1.1	Product Identification:		Dylon Cold Water Dye nos. 02 – 52.

Pack Size:			5.5 & 7.5 g tinlets.

1.2	Product use:			Domestic dyestuff for fabrics.

1.3	Company Identification:		Dylon International Ltd

WORSLEY BRIDGE ROAD

LOWER SYDENHAM

LONDON SE26 5HD

## UNITED KINGDOM

## TEL	 	(+ 44) 020 8663 4801

## FAX		(+ 44) 020 8658 6735

E-MAIL		DylonInt@dylon.co.uk

1.4	Emergency Telephone No:	0870 600 6266

_________________________________________________________________________________________________________

2.	composition/information on ingredients

The individual tins may contain the following hazardous ingredients in concentrations greater than 1 %.

Ingredient		  		EINECS No:	  	CAS No: 	  	Classification

C.I. Reactive Yellow 86      		274-956-4       		70865-29-1            	Xi: Irritant R43-36

C.I. Reactive Yellow 111    		273-336-0       		68957-39-1	    	Xi: Irritant R43

C.I. Reactive Yellow 125    		268-974-1      		68155-62-4	    	Xi: Irritant R43

C.I. Reactive Orange 4      		274-701-7       		70616-90-9            	Xn: Harmful R42/43

C.I. Reactive Orange 64    		280-744-2       		83763-57-9             	Xn: Harmful R42/43

C.I. Reactive Orange 86    		260-696-9       		57359-00-9             	Xn: Harmful R42/43

C.I. Reactive Red 2           		241-776-2       		17804-49-8             	Xi: Irritant R43-36

C.I. Reactive Red 11	      		260-832-7       		57583-69-4             	Xi:  Irritant R43

C.I. Reactive Blue 4          		236-363-9       		13324-20-4             	Xi:  Irritant R43

C.I. Reactive Blue 81	     		 ----------          		75030-18-1             	Xi:  Irritant R43

C.I. Reactive Blue 109      		274-959-0      		70865-31-5	   	Xi:  Irritant R43

________________________________________________________________________________________________________

3.	HAZARDS IDENTIFICATION

HARMFUL – May cause sensitization by inhalation and skin contact. Irritating to eyes and skin.

_________________________________________________________________________________________________________

4.	FIRST AID MEASURES (SYMPTOMS & ACTION)

Eye contact			Contact with eyes will cause irritation and redness. The eyes may water profusely.

Promptly rinse eyes with plenty of water for 15 Minutes. Consult a doctor.

Skin contact			Contact with skin may cause irritation.

Remove all contaminated clothing. Wash skin thoroughly with soap and water.

Inhalation			High dust concentrations may cause coughing and wheezing.

Remove exposed person to fresh air. Should any symptoms persist, seek medical advice.

Ingestion			Ingestion may cause irritation to the digestive tract, diarrhoea and nausea.

Do not induce vomiting. Rinse mouth out with water. Drink plenty of milk or water. In the event of significant ingestion or if any symptoms persist, seek medical advice.

_________________________________________________________________________________________________________

5.	FIRE FIGHTING MEASURES

Extinguishing Media       	Use extinguishing media appropriate to surrounding fire.

Exposure Hazards		May form flammable dust / air mixture.

Protective Equipment	     	Wear full protective clothing and self-contained breathing equipment.

_________________________________________________________________________________________________________

6.	ACCIDENTAL RELEASE MEASURES

Take precautions to avoid creating and breathing dust (see section 8). Absorb any spilled material immediately onto inert material i.e. sand or vermiculite and transfer into labelled containers for disposal (see section 13). Small spillages may be flushed away with copious amounts of water.

_________________________________________________________________________________________________________

7.	HANDLING AND STORAGE

Handling	When handling this product, observe good standards of industrial hygiene to avoid swallowing, inhaling dust and eye or skin contact by the use of suitable personnel protective equipment (PPE).  It is advisable always to wear hand protection when handling chemicals.

Storage				Store in a cool, dry, well-ventilated area.

_________________________________________________________________________________________________________

8.	EXPOSURE CONTROLS/ PERSONAL PROTECTION

8.1	Workplace Exposure Limit Values            Reactive dyes, of which this product is one type, have been reported as causing instances of respiratory sensitization. There is, however, limited or no information which would enable in-house exposure limits to be set. Under these circumstances the inhalation of dust must be avoided or exposure levels reduced to be as low as reasonably practicable. Persons with known respiratory or skin allergies should avoid using or take special precautions when using this product.

8.2      Exposure Controls

Respiratory Protection		None required under normal i.e. domestic use. Wear a suitable mask if handling is

likely to raise dust. This product should be weighed under conditions of Local

Exhaust Ventilation (LEV).

Hand Protection			Wear rubber, nitrile or butile gloves.

Eye Protection			None required under normal use. Where large amounts are being handled, wear

chemical resistant safety glasses or goggles.

Eyebaths should be provided at places where accidental exposure may occur.

Skin Protection	           		Wear protective clothing.

_________________________________________________________________________________________________________

9.	PHYSICAL AND CHEMICAL PROPERTIES

Appearance                                Coloured powder

Odour                                          Barely perceptible odour

pH                                                6.5 – 7.5 (10 g/l)

Solubility	                          Water – Highly soluble

_________________________________________________________________________________________________________

10	STABILITY AND REACTIVITY

Stable under normal and intended uses and applications.  Hazardous polymerisation will not occur.

Conditions to avoid	 		None known.

Materials to avoid	 		Strong acids.

Hazardous decomposition products	Contact with acids liberates toxic gas.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

11.	TOXICOLOGICAL INFORMATION

Acute Oral Toxicity		LD50 value :  > 2000 mg/kg (oral, rat – estimated from components)

Skin Irritation			Irritant, sensitization may also occur.

Eye Irritation			Irritant

_________________________________________________________________________________________________________

12.	ECOLOGICAL INFORMATION

Ecotoxicity	 		LC50  > 100 mg/l. [96hr, Fish].

No ecological testing has been carried out on this product, the data is based on the properties of the main components.

_________________________________________________________________________________________________________

13.	DISPOSAL CONSIDERATIONS

Disposal should be in accordance with all national and local authority regulations.

_________________________________________________________________________________________________________

14.	TRANSPORT INFORMATION

Proper shipping name		Not classified as dangerous in the meaning of transport regulations.

UN Number		--					IMDG / IMO		--

#### ADR/RID		--					IATA/ICAO		--

_________________________________________________________________________________________________________

15.	REGULATORY INFORMATION

Classified according to Directives 67/548/EEC and 1999/45/EC and their various amendments

Warning Symbols:	St. Andrew’s Cross

Warning Words:		HARMFUL

Risk Phrases:		R42/43		May cause sensitization by inhalation and skin contact.

R36/38		Irritating to eyes and skin.

Safety Phrases:		S2		Keep out of the reach of children.

S22 		Do not breathe dust.

S24		Avoid contact with skin.

S26	In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S37		Wear suitable gloves.

S46	If swallowed, seek medical advice immediately and show this container or label.

_________________________________________________________________________________________________________

16.	OTHER INFORMATION

The information in this Safety Data Sheet is given in good faith. The information herein is accurate to the best of our knowledge and represents the most up to date information available to us. Products should be handled and used in accordance with the advice given. The information in this Safety Data Sheet does not constitute an assessment of workplace risks.

Key to R Phrases – Section 2

R36 – Irritating to eyes.

R42/43 – May cause sensitization by inhalation and skin contact.

R43 – May cause sensitization by skin contact.

#### Recommended uses and restrictions

This product is intended for use as a dyestuff for fabrics as per the instruction leaflets/labels.

Revision Details: Replaces Issue CWD 004 Revision 1 dated 8.11.2002. _________________________________________________________________________________________________________