MATERIAL SAFETY DATA SHEET

_________________________________________________________________________________________________________

1.	IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY

1.1	Product Identification:		Dylon Multi-Purpose Dye Nos 1 – 65

Pack Size:			4.5 & 5.5 g tinlets.

1.2	Product use:			Domestic dyestuff for fabrics.

1.3	Company Identification:		Dylon International Ltd

WORSLEY BRIDGE ROAD

LOWER SYDENHAM

LONDON SE26 5HD

## UNITED KINGDOM

## TEL	 	(+ 44) 020 8663 4801

## FAX		(+ 44) 020 8658 6735

E-MAIL		DylonInt@dylon.co.uk

1.4	Emergency Telephone No:	0870 600 6266

_________________________________________________________________________________________________________

2.	composition/information on HAZARDOUS ingredients

Ingredient			      EINECS No:	  CAS No: 	  Concentration Range            Classification

C.I. Direct Black 22		      229-326-3                6473-13-8                 1 – 20 %                      Xi: Irritant R36

(MP8 & MP21 only)

C.I. Acid Red 88		      216-760-3                1658-56-6                   1 – 20 %	                      Xi: Irritant R36

(MP5, 9, 32 and 44 only)

C.I. Acid Orange 116                              ---                      12220-10-9	           1 – 5 %	                           R52/53

(MP34 Only)

C.I. Acid Blue 25                              229-068-1                6408-78-2	           5 – 20 %	                           R52/53

(MP26 & MP34 Only)

C.I. Acid Blue 80                              224-748-4                 4474-24-2	           1 – 5 %	               	 R52/53

(MP17 Only)

C.I. Acid Blue 113		      222-111-5	             3351-05-1                   1 – 10%		          N: R51/53

(MP17, MP19, MP26 & MP62 only)

C.I. Acid Green 25                           224-546-6                4403-90-1	           5 – 15 %	                          R52/53

(MP25 & MP62 Only)

C.I. Acid Violet 17                            223-942-6                4129-84-4                    1 – 5 %	                          R52/53

(MP15 & MP16 Only)

C.I. Acid Brown 283                        274-491-7               70236-61-2                  15 – 25 %	                          R52/53

(MP7 Only)

_________________________________________________________________________________________________________

3.	HAZARDS IDENTIFICATION

No significant hazard.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

4.	FIRST AID MEASURES (SYMPTOMS & ACTION)

Eye contact			Contact with eyes may cause slight irritation.

Promptly rinse eyes with plenty of water for 15 Minutes.

Skin contact			Contact with skin may cause slight irritation.

Wash skin thoroughly with soap and water.

Inhalation			High dust concentrations may cause coughing and wheezing.

Remove exposed person to fresh air. Should any symptoms persist, seek medical advice.

Ingestion			Ingestion may cause irritation to the digestive tract, diarrhoea and nausea.

Do not induce vomiting. Rinse mouth out with water. Drink plenty of milk or water. In the event of significant ingestion or if any symptoms persist, seek medical advice.

_________________________________________________________________________________________________________

5.	FIRE FIGHTING MEASURES

Extinguishing Media       		Use extinguishing media appropriate to surrounding fire.

Exposure Hazards	     		May form flammable dust / air mixture.

Protective Equipment	     		Wear full protective clothing and self-contained breathing equipment.

_________________________________________________________________________________________________________

6.	ACCIDENTAL RELEASE MEASURES

Take precautions to avoid creating and breathing dust (see section 8). Absorb any spilled material immediately onto inert material i.e. sand or vermiculite and transfer into labelled containers for disposal (see section 13). Small spillages may be flushed away with copious amounts of water.

_________________________________________________________________________________________________________

7.	HANDLING AND STORAGE

Handling	When handling this product, observe good standards of industrial hygiene to avoid swallowing, inhaling dust and eye or skin contact by the use of suitable personnel protective equipment (PPE). It is advisable always to wear hand protection when handling chemicals.

Storage		Store in a cool, dry, well-ventilated area.

_________________________________________________________________________________________________________

8.	EXPOSURE CONTROLS/ PERSONAL PROTECTION

  - Exposure Limit Values      No Workplace Exposure Limits have been assigned to the individual components

of this product.  In the absence of a specific limit, dust concentrations should be controlled and kept below

the following limits:

WES (UK)				Long term 8 hr TWA		Short term 15 min ref. period

ppm		mg/m³		ppm		mg/m³

Total 	inhalable dust			---		   10		  --- 		    ---

respirable dust		            ---		    4		  ---		    ---

8.2      Exposure Controls

Respiratory Protection		None required under normal i.e. domestic use. Wear a suitable mask if handling is

likely to raise dust. This product should be weighed under conditions of Local

Exhaust Ventilation (LEV).

Hand Protection			Wear rubber or chemical resistant gloves.

Eye Protection			None required under normal use. Where large amounts are being handled, wear

chemical resistant safety glasses or goggles.

Eyebaths should be provided at places where accidental exposure may occur.

Skin Protection	           		None required under normal use.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

9.	PHYSICAL AND CHEMICAL PROPERTIES

Appearance                     Coloured powder

Odour                              Barely perceptible

pH                                    7 – 9 (10 g/l)

Solubility – Water            Highly soluble

_________________________________________________________________________________________________________

10	STABILITY AND REACTIVITY

Stable under normal and intended uses and applications.  Hazardous polymerisation will not occur.

Conditions to avoid	 		None known.

Materials to avoid	 		Strong acids.

Hazardous decomposition products	Contact with acids liberates toxic gas.

_________________________________________________________________________________________________________

11.	TOXICOLOGICAL INFORMATION

Acute Oral Toxicity	LD50 value : All components >2000 mg/kg (oral, rat)

Skin Irritation		Non – irritant

Eye Irritation		Non – irritant

_________________________________________________________________________________________________________

12.	ECOLOGICAL INFORMATION

C.I. Acid Orange 116	LC50  10 - 100 mg/l. [48hr, Rainbow Trout]

C.I. Acid Blue 25	LC50  32 mg/l. [48hr, Rainbow Trout]

C.I. Acid Blue 80	LC50  Approx. 75 mg/l. [48hr, Rainbow Trout]

C.I. Acid Blue 113   	LC50  1 – 10 mg/l. [96hr, Rainbow Trout].

C.I. Acid Violet 17	LC50  10 – 100 mg/l. [48hr, fish].

C.I. Acid Green 25  	LC50  14.3 mg/l. [96hr, Guppies].

C.I. Acid Brown 283	LC50  16 mg/l. [96hr, Zebra Fish].

_________________________________________________________________________________________________________

13.	DISPOSAL CONSIDERATIONS

Disposal should be in accordance with all national and local authority regulations.

_________________________________________________________________________________________________________

14.	TRANSPORT INFORMATION

Proper shipping name		Not classified as dangerous in the meaning of transport regulations

UN Number		--					IMDG / IMO		--

#### ADR/RID		--					IATA/ICAO		--

_________________________________________________________________________________________________________

15.	REGULATORY INFORMATION

Classified according to Directives 67/548/EEC and 1999/45/EC and their various amendments

Warning Symbols:		None

Warning Words:			None

Risk Phrases:			None

Safety Phrases:			None

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

16.	OTHER INFORMATION

The information in this Safety Data Sheet is given in good faith. The information herein is accurate to the best of our knowledge and represents the most up to date information available to us. Products should be handled and used in accordance with the advice given. The information in this Safety Data Sheet does not constitute an assessment of workplace risks.

Key to R Phrases – Section 2

R36 – Irritating to eyes.

R51/53 – Toxic to aquatic organisms, may cause long term adverse effects in the aquatic environment.

R52/53 – Harmful to aquatic organisms, may cause long term adverse effects in the aquatic environment.

#### Recommended uses and restrictions

This product is intended for use as a dyestuff for fabrics as per the instruction leaflets/labels.

Abbreviations:

CAS = Chemical Abstract Service (Section 2)

C.I. = Colour Index  (Section 2)

EINECS = European Inventory of Existing Commercial Chemical Substances  (Section 2)

LC = Lethal Concentration  (Section 12)

LD = Lethal Dose  (Section 11)

TWA = Time Weighted Average  (Section 8)

WES = Workplace Exposure Standard  (Section 8)

Revision Details: Replaces Issue MPD 01 Rev. no. 7 dated 23.01.2007. _________________________________________________________________________________________________________