POLYPIPE BUILDING PRODUCTS LTD SAFETY DATA SHEET

Date: June 2002

Code: Cleaning Fluid

Page: 1 of 4

- IDENTIFICATION PRODUCT NAME: Polypipe Cleaning Fluid CF250 SUPPLIER: POLYPIPE BUILDING PRODUCTS LTD BROOMHOUSE LANE DONCASTER DN12 1ES TELEPHONE: 01709 770000 EMERGENCY: 01785 255141 The following information is based upon our current knowledge and experience of our products and is not exhaustive. It applies to the product range outlined above. The potential hazards identified, together with the recommended precautions for storage, handling and use, give the basic information for conducting workplace risk assessments and adequate control measures under the Control of Substances Hazardous to Health Regulations 1999 and Approved Code of Practice.

2. COMPOSITION / INFORMATION ON INGREDIENTS

Butanone (Other name -Methyl Ethyl Ketone) CAS No#78-93-3 Content : 60 – 100% Hazardous Ingredients : Xi Risk : 11, 36, 66, 67

3. HAZARDS IDENTIFICATION

Highly Flammable. Irritating to the eyes. Repeated exposure may cause skin dryness or cracking. Vapours may cause drowsiness and dizziness.

4. FIRST AID MEASURES

Eye contact: Rinse with clean water immediately. Seek medical advice. Continue to rinse. Skin contact: Remove contaminated clothing. Rinse immediately with plenty of water. Get medical treatment if irritation persists after washing. Ingestion: Do not induce vomiting. Obtain medical attention immediately. Inhalation : Move the exposed person to fresh air immediately. Get medical attention.

5. FIRE FIGHTING MEASURES

Extinguishing Media: Stop flow of material to fire. Use sand or earth. Fire can be extinguished using foam or dry powder. Do not use water jet Special Fire Fighting Procedures: Cool containers exposed to flames with water until well after the fire is out. Hazardous Decomposition Products: Carbon Monoxide (CO), Carbon Dioxide (CO2)

SAFETY DATA SHEET

Date: June 2002

Code: Cleaning Fluid

Page: 2 of 4

6. ACCIDENTAL RELEASE MEASURES

Spill Cleanup Methods : Extinguish all ignition sources. Avoid sparks, flames, heat and smoking. Ventilate. Absorb in vermiculite, dry sand or earth and place into containers. Pick up using spark proof tools and place in a sealable metal container pending disposal. Container to be labelled with contents description. Do not contaminate water sources or sewer.

7. HANDLING AND STORAGE

Usage Precautions : Avoid spilling, skin and eye contact. Avoid inhalation of vapours. Keep away from heat, sparks and open flame. Static electricity and formulation of sparks must be prevented. Storage Precautions : Store in closed containers in a dry, ventilated place where the temperature is between 5 – 30°C. Store separately from oxidising material. Storage Criteria : Flammable liquid storage.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient Name Cas No# OEL 8Hr TWA 15min STEL Butanone 78-93-3 OES 200ppm(Sk) 300ppm(Sk) OES taken from HSE Guidance Note EH40/2001, Occupational Exposure Limits Ventilation : Provide adequate general and local exhaust ventilation Respirators : If ventilation is insufficient, suitable respiratory protection must be provided with organic vapour filter Protective Gloves : Use protective gloves recommended by glove supplier for Butanone Eye Protection : Wear splash proof goggles or visor Other Protection : Wear suitable chemical suit in the event of splashing or skin contamination

9. PHYSICAL AND CHEMICAL PROPERTIES *

Appearance: Clear, volatile liquid Odour/Taste: Ketonic Specific Gravity 0.8 g/ml Flash Point - 5°C (closed cup) Auto ignition temperature >514°C Solubility Soluble in water Flammability Limit (lower) 1.8% Flammability Limit (upper) 11.5% Boiling point 79 -80°C Vapour Pressure 78 mm Hg at 20°C

10. STABILITY AND REACTIVITY

Conditions to avoid : Avoid contact with strong oxidising agents Hazardous Decomp. Products: Fire or high temperatures create: Carbon monoxide (CO). Carbon Dioxide (CO2).

SAFETY DATA SHEET

Date: June 2002

Code: Cleaning Fluid

Page: 3 of 4

11. TOXICOLOGICAL INFORMATION

Inhalation : Vapour may affect central nervous system and cause headache, discomfort, vomiting or intoxication, leading to unconsciousness Ingestion : Liquid irritates mucous membranes and may cause abdominal pain if swallowed Skin : Liquid may irritate skin. Prolonged or repeated contact leads to drying of the skin. Eyes : Irritating to the eyes

12. ECOLOGICAL INFORMATION

Contamination of both aquatic and terrestrial environments should be avoided. Once the solvent has evaporated there is no residual risk to the environment. The concentration of solvent vapour in the atmosphere must meet limits set under the Environmental Protection Act 1990.

13. WASTE DISPOSAL

Disposal Methods: Waste product comes within the scope of The Special Waste Regulations 1996. Emptied containers and those containing scrap or spilled material should be re-sealed or lidded so that vapour is confined to the container. Waste containers should be treated as full containers. Use authorised disposal company for disposal.

14. TRANSPORT INFORMATION

# Road Transport UN No. 1193 ADR Class No. Class 3, Flammable Liquid ADR Item No. 3O(b) # Rail Transport RID Class. 3 3(b) # Sea Transport IMDG Class 3.2 IMDG Pack Group II

15. REGULATORY INFORMATION

# Label for Supply Highly Flammable, Irritant # Risk Phrases R11 Highly flammable R36 Irritating to eyes R66 Repeated exposure may cause skin dryness or cracking R67 Vapours may cause drowsiness and dizziness # Safety Phrases S16 Keep away from sources of ignition S37 Wear suitable gloves S51 Use only in well ventilated areas S24/25 Avoid contact with skin and eyes # UK Regulatory References Chemicals (Hazard Information & Packaging for Supply) Regulations 1994

## SAFETY DATA SHEET

Date: June 2002

Code: Cleaning Fluid

Page: 4 of 4

16. OTHER INFORMATION

This product should be used as directed. Recipients of the product must take responsibility for observing existing laws and regulations. This Safety Data Sheet was complied using the current safety information supplied by the distributors of the component materials. This Safety Data Sheet, in CHIP format, supersedes all previous issues, and users are cautioned to ensure it is current as of June 2002.